Instructions:

Simply drag and drop the appropriate sign/symbol/number for your lab in the correct area on the publisher document. An example is below.

Type building abbreviation ( JST, WSB, PSRC, etc,..) and room number here

Building name and room number

- Custodial/Maintenance

Drag  Custodial/Maintenance entry circles here

Note: if you place the red “no entry” Custodial will not clean or empty trash in the lab

The symbol denotes the total lab hazards so use the highest hazard symbols for each category

NFPA/Hazards Instructions

Use the numbers for the NFPA symbol (example below).  This denotes the chemical hazards in your specific lab. See the provided list of chemicals (end of document) and their NFPA hazard rating to make this decision.  Drag the appropriate hazard symbols for your lab to the area below the NFPA symbol

NFPA symbol

Hazard symbols

Some labs will have just a few symbols. In this case use the larger symbol size

The diamond represents the total hazards so use the highest values for each hazard section

Red = flammability

Blue = health

Yellow = reactivity

White = special/specific hazards

3

1

0

An example:  a lab that contained only acetone would look like the example below:

If the lab contained potassium cyanide:

0

3

0

3

3

0

A lab that had both Acetone and potassium cyanide would be:

NFPA Special/Specific Hazard symbols:

OX

Oxidizer

ALK

Alkaline

ACID

Acids

No water

Hazard symbols:

Toxic	     Flammable	  Corrosive 	Gas cylinders

Radioactive	       Biohazard

- Personal Protection Equipment (PPE) requirements

Place PPE symbols here

- Contact Information

Type in contact information for the PI, safety officer and any other relevant contact.  At least one after hours contact for the specific lab should be included.